1.3 DE10-Standard引脚列表

表1.1 时钟输入引脚配置

| 信号名 | FPGA引脚号 | 描述 | I/O 标准 |
| --- | --- | --- | --- |
| CLOCK_50 | PIN_AF14 | 50 MHz时钟输入 | 3.3V |
| CLOCK2_50 | PIN_AA16 | 50 MHz时钟输入 | 3.3V |
| CLOCK3_50 | PIN_Y26 | 50 MHz时钟输入 | 3.3V |
| CLOCK4_50 | PIN_K14 | 50 MHz时钟输入 | 3.3V |
| HPS_CLOCK1_25 | PIN_D25 | 25 MHz时钟输入 | 3.3V |
| HPS_CLOCK2_25 | PIN_F25 | 25 MHz时钟输入 | 3.3 |

表1.2 按钮开关引脚配置

| 组件名 | FPGA引脚号 | 描述 | I/O标准 |
| --- | --- | --- | --- |
| KEY[0] | PIN_AJ4 | Push-button[0] | 3.3V |
| KEY[1] | PIN_AK4 | Push-button[1] | 3.3V |
| KEY[2] | PIN_AA14 | Push-button[2] | 3.3V |
| KEY[3] | PIN_AA15 | Push-button[3] | 3.3V |

表1.3 拨码开关引脚配置

| 组件名 | FPGA引脚号 | 描述 | I/O标准 |
| --- | --- | --- | --- |
| SW[0] | PIN_AB30 | Slide Switch[0] | 取决于JP3 |
| SW[1] | PIN_Y27 | Slide Switch[1] | 取决于JP3 |
| SW[2] | PIN_AB28 | Slide Switch[2] | 取决于JP3 |
| SW[3] | PIN_AC30 | Slide Switch[3] | 取决于JP3 |
| SW[4] | PIN_W25 | Slide Switch[4] | 取决于JP3 |
| SW[5] | PIN_V25 | Slide Switch[5] | 取决于JP3 |
| SW[6] | PIN_AC28 | Slide Switch[6] | 取决于JP3 |
| SW[7] | PIN_AD30 | Slide Switch[7] | 取决于JP3 |
| SW[8] | PIN_AC29 | Slide Switch[8] | 取决于JP3 |
| SW[9] | PIN_AA30 | Slide Switch[9] | 取决于JP3 |

表1.4 LED引脚配置

| 组件名 | FPGA引脚号 | 描述 | I/O标准 |
| --- | --- | --- | --- |
| LEDR[0] | PIN_AA24 | LED [0] | 3.3V |
| LEDR[1] | PIN_AB23 | LED [1] | 3.3V |
| LEDR[2] | PIN_AC23 | LED [2] | 3.3V |
| LEDR[3] | PIN_AD24 | LED [3] | 3.3V |
| LEDR[4] | PIN_AG25 | LED [4] | 3.3V |
| LEDR[5] | PIN_AF25 | LED [5] | 3.3V |
| LEDR[6] | PIN_AE24 | LED [6] | 3.3V |
| LEDR[7] | PIN_AF24 | LED [7] | 3.3V |
| LEDR[8] | PIN_AB22 | LED [8] | 3.3V |
| LEDR[9] | PIN_AC22 | LED [9] | 3.3V |

表1.5 七段数码管引脚配置

| 信号名 | FPGA引脚号 | 描述 | I/O标准 |
| --- | --- | --- | --- |
| HEX0[0] | PIN_W17 | Seven Segment Digit 0[0] | 3.3V |
| HEX0[1] | PIN_V18 | Seven Segment Digit 0[1] | 3.3V |
| HEX0[2] | PIN_AG17 | Seven Segment Digit 0[2] | 3.3V |
| HEX0[3] | PIN_AG16 | Seven Segment Digit 0[3] | 3.3V |
| HEX0[4] | PIN_AH17 | Seven Segment Digit 0[4] | 3.3V |
| HEX0[5] | PIN_AG18 | Seven Segment Digit 0[5] | 3.3V |
| HEX0[6] | PIN_AH18 | Seven Segment Digit 0[6] | 3.3V |
| HEX1[0] | PIN_AF16 | Seven Segment Digit 1[0] | 3.3V |
| HEX1[1] | PIN_V16 | Seven Segment Digit 1[1] | 3.3V |
| HEX1[2] | PIN_AE16 | Seven Segment Digit 1[2] | 3.3V |
| HEX1[3] | PIN_AD17 | Seven Segment Digit 1[3] | 3.3V |
| HEX1[4] | PIN_AE18 | Seven Segment Digit 1[4] | 3.3V |
| HEX1[5] | PIN_AE17 | Seven Segment Digit 1[5] | 3.3V |
| HEX1[6] | PIN_V17 | Seven Segment Digit 1[6] | 3.3V |
| HEX2[0] | PIN_AA21 | Seven Segment Digit 2[0] | 3.3V |
| HEX2[1] | PIN_AB17 | Seven Segment Digit 2[1] | 3.3V |
| HEX2[2] | PIN_AA18 | Seven Segment Digit 2[2] | 3.3V |
| HEX2[3] | PIN_Y17 | Seven Segment Digit 2[3] | 3.3V |
| HEX2[4] | PIN_Y18 | Seven Segment Digit 2[4] | 3.3V |
| HEX2[5] | PIN_AF18 | Seven Segment Digit 2[5] | 3.3V |
| HEX2[6] | PIN_W16 | Seven Segment Digit 2[6] | 3.3V |
| HEX3[0] | PIN_Y19 | Seven Segment Digit 3[0] | 3.3V |
| HEX3[1] | PIN_W19 | Seven Segment Digit 3[1] | 3.3V |
| HEX3[2] | PIN_AD19 | Seven Segment Digit 3[2] | 3.3V |
| HEX3[3] | PIN_AA20 | Seven Segment Digit 3[3] | 3.3V |
| HEX3[4] | PIN_AC20 | Seven Segment Digit 3[4] | 3.3V |
| HEX3[5] | PIN_AA19 | Seven Segment Digit 3[5] | 3.3V |
| HEX3[6] | PIN_AD20 | Seven Segment Digit 3[6] | 3.3V |
| HEX4[0] | PIN_AD21 | Seven Segment Digit 4[0] | 3.3V |
| HEX4[1] | PIN_AG22 | Seven Segment Digit 4[1] | 3.3V |
| HEX4[2] | PIN_AE22 | Seven Segment Digit 4[2] | 3.3V |
| HEX4[3] | PIN_AE23 | Seven Segment Digit 4[3] | 3.3V |
| HEX4[4] | PIN_AG23 | Seven Segment Digit 4[4] | 3.3V |
| HEX4[5] | PIN_AF23 | Seven Segment Digit 4[5] | 3.3V |
| HEX4[6] | PIN_AH22 | Seven Segment Digit 4[6] | 3.3V |
| HEX5[0] | PIN_AF21 | Seven Segment Digit 5[0] | 3.3V |
| HEX5[1] | PIN_AG21 | Seven Segment Digit 5[1] | 3.3V |
| HEX5[2] | PIN_AF20 | Seven Segment Digit 5[2] | 3.3V |
| HEX5[3] | PIN_AG20 | Seven Segment Digit 5[3] | 3.3V |
| HEX5[4] | PIN_AE19 | Seven Segment Digit 5[4] | 3.3V |
| HEX5[5] | PIN_AF19 | Seven Segment Digit 5[5] | 3.3V |
| HEX5[6] | PIN_AB21 | Seven Segment Digit 5[6] | 3.3V |

表1.6  I2C总线引脚配置

| 信号名 | FPGA引脚号 | 描述 | I/O标准 |
| --- | --- | --- | --- |
| FPGA_I2C_SCLK | PIN_Y24 | FPGA I2C时钟 | 3.3V |
| FPGA_I2C_SDAT | PIN_Y23 | FPGA I2C数据 | 3.3V |
| HPS_I2C1_SCLK | PIN_E23 | 第一个HPS I2C控制器的I2C时钟 | 3.3V |
| HPS_I2C1_SDAT | PIN_C24 | 第一个HPS I2C控制器的I2C数据 | 3.3V |
| HPS_I2C2_SCLK | PIN_H23 | 第二个HPS I2C控制器的I2C时钟 | 3.3V |
| HPS_I2C2_SDAT | PIN_A25 | 第二个HPS I2C控制器的I2C数据 | 3.3V |

表1.7 VGA引脚配置

| 信号名 | FPGA引脚号 | 描述 | I/O标准 |
| --- | --- | --- | --- |
| VGA_R[0] | PIN_AK29 | VGA Red[0] | 3.3V |
| VGA_R[1] | PIN_AK28 | VGA Red[1] | 3.3V |
| VGA_R[2] | PIN_AK27 | VGA Red[2] | 3.3V |
| VGA_R[3] | PIN_AJ27 | VGA Red[3] | 3.3V |
| VGA_R[4] | PIN_AH27 | VGA Red[4] | 3.3V |
| VGA_R[5] | PIN_AF26 | VGA Red[5] | 3.3V |
| VGA_R[6] | PIN_AG26 | VGA Red[6] | 3.3V |
| VGA_R[7] | PIN_AJ26 | VGA Red[7] | 3.3V |
| VGA_G[0] | PIN_AK26 | VGA Green[0] | 3.3V |
| VGA_G[1] | PIN_AJ25 | VGA Green[1] | 3.3V |
| VGA_G[2] | PIN_AH25 | VGA Green[2] | 3.3V |
| VGA_G[3] | PIN_AK24 | VGA Green[3] | 3.3V |
| VGA_G[4] | PIN_AJ24 | VGA Green[4] | 3.3V |
| VGA_G[5] | PIN_AH24 | VGA Green[5] | 3.3V |
| VGA_G[6] | PIN_AK23 | VGA Green[6] | 3.3V |
| VGA_G[7] | PIN_AH23 | VGA Green[7] | 3.3V |
| VGA_B[0] | PIN_AJ21 | VGA Blue[0] | 3.3V |
| VGA_B[1] | PIN_AJ20 | VGA Blue[1] | 3.3V |
| VGA_B[2] | PIN_AH20 | VGA Blue[2] | 3.3V |
| VGA_B[3] | PIN_AJ19 | VGA Blue[3] | 3.3V |
| VGA_B[4] | PIN_AH19 | VGA Blue[4] | 3.3V |
| VGA_B[5] | PIN_AJ17 | VGA Blue[5] | 3.3V |
| VGA_B[6] | PIN_AJ16 | VGA Blue[6] | 3.3V |
| VGA_B[7] | PIN_AK16 | VGA Blue[7] | 3.3V |
| VGA_CLK | PIN_AK21 | VGA Clock | 3.3V |
| VGA_BLANK_N | PIN_AK22 | VGA BLANK | 3.3V |
| VGA_HS | PIN_AK19 | VGA H_SYNC | 3.3V |
| VGA_VS | PIN_AK18 | VGA V_SYNC | 3.3V |
| VGA_SYNC_N | PIN_AJ22 | VGA SYNC | 3.3V |

表1.8 LCD模块引脚配置

| 信号名 | FPGA引脚号 | 描述 | I/O标准 |
| --- | --- | --- | --- |
| HPS_LCM_D_C | PIN_C18 | HPS LCM Data bit is Data/Command | 3.3V |
| HPS_LCM_RST_N | PIN_E17 | HPS LCM Reset | 3.3V |
| HPS_LCM_SPIM_CLK | PIN_A23 | SPI Clock | 3.3V |
| HPS_LCM_SPIM_MOSI | PIN_C22 | SPI Master Output /Slave Input | 3.3V |
| HPS_LCM_SPIM_SS | PIN_H20 | SPI Slave Select | 3.3V |